## Proposal: Booth Show Control Network Upgrade

Here is the straightforward breakdown of the hardware layout and gear we need to build our isolated booth network, completely replacing the broken legacy architectural control system with our new Stream Deck setup.

### What the Parts Do (Simplified Breakdown)

- Mini PC (Beelink SEi12): The brain of the operation. This runs the Bitfocus Companion software. It listens for button presses from the Stream Decks on the stage/doors and translates them into commands for our Element 2 light board, sound console, and projectors.

- 16-Port Network Switch (Netgear GS316EP): The central crossroads. It connects all our booth gear together on its own dedicated network. Because it has PoE+ ports, it sends power directly down the ethernet lines to the Stream Decks so they don’t need local wall outlets.

- Power Conditioner (Furman M-8z): Surge protection and power distribution. It mounts right into the rack to power the switch and mini PC safely. It also features front-facing USB ports to charge a phone or booth tablet.

- Stealth Router (GL.iNet Opal): Our internet gateway. It bridges the rack to the school network wall jack for software updates and remote control, but isolates all our actual gear behind a strict firewall so IT can't scan it. It's tiny, pocket-sized, and easy to unplug instantly.

- 6U Wall-Mount Cabinet (StarTech): The steel box that holds all this equipment cleanly on the booth wall or counter. It features a lockable glass door.

- 1U Vented Shelf (AC Infinity): A sturdy metal rack shelf to securely hold the mini PC and the pocket router.

- Blank Patch Panel & Keystone Couplers (Cable Matters): The termination wall. The stiff ethernet lines pulled from the stage walls end here permanently so they don't get bent or snapped. The inline couplers are double-sided barrel jacks that let us snap the lines into the panel without punch-downs.

- 1U Brush Cable Manager (DuraRack): A simple panel with a built-in brush slot that hides the excess slack of our jumper cables to keep the rack looking completely organized.

### Estimated Parts List & Costs

| Component | Exact Model | Main Purpose | Estimated Cost |
| --- | --- | --- | --- |
| Mini PC | Beelink SEi12 (Intel i5, 16GB RAM, 500GB SSD) | Runs Bitfocus Companion | $350.00 |
| Network Switch | Netgear GS316EP 16-Port PoE+ Easy Smart Switch | Connects & powers devices | $240.00 |
| Power Conditioner | Furman Merit Series M-8z (1U Rackmount) | Surge protection & power delivery | $170.00 |
| Travel Router | GL.iNet GL-SFT1200 (Opal) Travel Router | Isolated internet bridge | $40.00 |
| Rack Enclosure | StarTech.com 6U Wall Mount Network Cabinet (16") | Securely houses all system gear | $160.00 |
| Rack Shelf | AC Infinity 1U Universal Vented Shelf (10" Depth) | Holds the Mini PC & router | $25.00 |
| Patch Panel | Cable Matters 24-Port Blank Keystone 1U Panel | Anchors incoming stage cables | $18.00 |
| Keystone Jacks | Cable Matters Cat6 RJ45 Inline Couplers (20-Pack) | Quick-connect barrel inserts | $32.00 |
| Cable Manager | DuraRack 1U Horizontal Brush Strip Panel | Organizes short patch cables | $15.00 |
| Total Project Cost |  |  | ~$1,050.00 |

### Physical Rack Layout Diagram

The components will stack vertically inside the 6U cabinet from top to bottom like this to maximize heat ventilation, keep the center of gravity low, and protect the main power switch from accidental bumps:

U1: Furman M-8z Power Conditioner

U2: AC Infinity Rack Shelf (Holding Beelink Mini PC)

U3: Cable Matters 24-Port Blank Patch Panel

U4: DuraRack 1U Horizontal Brush Cable Manager

U5: Netgear GS316EP 16-Port Managed PoE+ Switch

U6: Open Space (For lower ventilation/future expansion)